INF2004 Embedded Systems Programming

Week 8 Tribunal 1 Template

Architecture Defence & Integration Readiness | AI ideation, live update, AI-vetted gap check

| Item | Details |
| --- | --- |
| Purpose | Defend Week 6 architecture, justify changes, and show readiness for partial integration. |
| Core workflow | Pre-submission AI ideation -> live Zoom-updated version -> final AI-vetted gap check |
| Assessment focus | Evidence, engineering judgement, negotiation, and ability to identify missing work |
| Submission format | Completed document plus Zoom recording and evidence links |

Important: this submission has THREE versions inside one document - Version A: Pre-tribunal AI ideation. Each buddy uses AI to ideate their subsystem position before Zoom. - Version B: Live Zoom-updated version. Design decisions are updated during the Zoom discussion through challenge, negotiation, and constraint response. - Version C: Final AI-vetted gap check. After discussion, AI is used as a reviewer to identify missing assumptions, risks, tests, or unclear decisions. - Marks are not awarded for blindly following AI. Marks are awarded for critical evaluation and human engineering judgement; meaning, I want to see you guys discuss during zoom.

# Team Information

| Item | Details |
| --- | --- |
| Group ID | CSC05 |
| Repository Link | https://github.com/lennoly/INF2004_Grp5 |

| Buddy | Student ID | Primary subsystem / persona |
| --- | --- | --- |
| Buddy 1 | 2500343 | Network Gatekeeper: WiFi, MQTT, telemetry |
| Buddy 2 | 2503109 | Real-Time Taskmaster: motors, encoders, PID |
| Buddy 3 | 2501660 | Track Purist: IR sensors, line following, barcode |
| Buddy 4 | 2503438 | Physics Watchdog: IMU, terrain, hump/motion events |
| Buddy 5 | 2501955 | Spatial Optimizer: ultrasonic, servo, obstacle profiling |

# Version A: Pre-Tribunal AI Ideation

Instructions • Each buddy must use AI before Zoom session to prepare subsystem-specific arguments, risks, and negotiation points. • The AI output must be critiqued. Identify what is useful, unrealistic, incomplete, or requires validation. • For Tribunal 1, compare against the Week 6 architecture freeze. For Tribunal 2, compare against actual integration evidence.

Evidence baseline: Week 6 review and lecturer feedback; source inspection at repository commit d5f5199. Planned tests are not recorded passes.

| Buddy | AI prompt used | Useful AI ideas | Weak / unrealistic AI ideas | Position before Zoom |
| --- | --- | --- | --- | --- |
| Buddy 1 | Act as a tribunal examiner. Our MQTT client runs inside the template cyw43 task (priority 4, above motion at 5) through the cyw43_utk_app_poll() hook, with non-blocking rings to the telemetry task. Since Week 6 we added Wi-Fi re-join (3 s after link loss, then every 5 s). What will the other subsystems challenge, what evidence do I need, and what should I concede? | Measure motion jitter during broker loss and AP re-join. Test both recovery paths separately. Check that full rings drop/count rather than block producers. | A separate MQTT task conflicts with current single-owner lwIP access. Request IDs alone do not provide retries or owner-confirmed application. An asynchronous join API does not prove bounded execution time. | Retain one cyw43/lwIP owner provisionally. Re-join now uses 5 s retries rather than the Week 6 5/10/20 s schedule; justify this change. Measure re-join cost and jitter <2 ms. Board link tests and reliable applied replies remain open. Publish all topic schemas. |
| Buddy 2 | Review a 50 Hz per-wheel speed PID on µT-Kernel woken by a cyclic handler at priority 5, with the template usb (3) and cyw43 (4) tasks above it and single-channel encoders (40 counts/rev). What will be challenged at integration, and which tests prove the motion layer is ready for others to depend on? | Measure control jitter, CPU load and execution maxima under load. encoder.c counts both edges but estimates speed from rising-to-rising periods averaged over four slots, addressing unequal slot duty in code. | Quadrature needs verified outputs/pins and an IRQ budget. A faster loop does not create fresh encoder data. Do not raise motion to priority 1 without measurements. Software motor inversion can be valid if the direction convention is consistent. | Keep the 20 ms loop and priority 5 provisionally. Validate the 4 mm completion threshold against nominal 5 mm/count, shortest half-period versus the 300 us filter, coast/reversal signs and braking. STOP latch, CANCELLED, velocity expiry and watchdog remain design/code gaps. |
| Buddy 3 | Our line follower uses two analogue IR sensors with PID steering. A third IR sensor reads Code 39 barcodes, sampled at 1 kHz in a timer ISR and decoded from the 29th element. What will the examiner challenge, and what are the integration failure modes (speed, turns, lighting, placement)? | The 300 mm/s limit applies at the barcode sensor, including turns. Test contrast with motors running, both reading directions, bends and post-U-turn placement. | Ranking bar widths does not solve analogue black/white thresholding or PWM noise. Synthetic frames do not prove track performance. A fourth analogue sensor needs a revised ADC allocation; an opposite-side digital sensor is a separate GPIO/ISR option. | Retain 1 kHz capture and early decoding provisionally. A cruise-command cap alone does not enforce sensor speed in turns. Measure x_bc/y_bc, implement the NFR2 bound and test A6/A11. Check course coverage after U-turns before changing the pin/ISR design. |
| Buddy 4 | The IMU fitted to our car is an LSM303D (0x1D on I2C0), not the GY-511 (LSM303DLHC) in our Week 6 design, and the driver has been ported. Hump height comes from a front-contact estimator using pitch and encoder distance with a 10 mm trigger. What does the hardware change break in our assumptions, and what must I test before integration? | Verify LSM303D identity, configuration and axes on the car. Re-measure pitch noise and magnetic behaviour. Check leading contact in each direction, measured L and wheel slip. | The prompt describes the Week 6 estimator, not current firmware: terrain.c still uses ds*sin(pitch) and a 6-degree trigger. A gyro-based filter is unsuitable; the part has no gyro. Simulation does not prove NFR6. | The driver port is in code, but the front-contact/10 mm design and freshness/fault contract are not. Reconcile them, then test real humps. The feedback cites 1.6-4.6 mm simulated error only. If ±5 mm fails, report NFR6 unmet and height unvalidated. Keep A7/A16 open. |
| Buddy 5 | Our HC-SR04 runs at 3.3 V on a servo. Week 6 treats OUT_OF_RANGE as free space only if the module gives a long echo with nothing in range (A9). Here is our first log [pasted]. What can I now claim, what is still unproven, and how will the obstacle and integration plan be challenged? | The screenshot has 4 VALID, 6 OUT_OF_RANGE and 0 FAULT readings. Explain the mixed target conditions; timeout-path widths are not completed echo pulses. Locking currently covers ping only. | The screenshot does not qualify 3.3 V operation or no-target clearance. Five pings at ≥60 ms spacing span ≥240 ms before the final echo, exceeding the 180 ms NFR5 budget. Forward travel must keep the sonar centred. | Validate A9 first, then A2/A14, A3, A6 and A7. Confirm servo movement and range accuracy; decide and retest the 5 V plus divider fallback if needed. Close transaction-lock/STOP/fault gaps and budget A10 scans, bypasses and re-plans. Keep an integrated baseline after each change. |

# Week 6 Architecture Defence Snapshot Before Zoom

Select only 5 to 7 important Week 6 architecture claims that affect integration readiness. Do not repeat the entire Week 6 document. Each row should defend one claim using evidence, remaining weakness, and the buddy responsible. One smart-toaster example is shown; replace it with your own project claim.

| Week 6 artefact | What the team claimed in Week 6 | Evidence available now | Still weak or untested | Buddy defending it |
| --- | --- | --- | --- | --- |
| Task table section 3; NFR1; A2 | Motion priority 5, woken every 20 ms; usb (3) and cyw43 (4) pre-empt it. A2 assumes jitter <2 ms; this was not a measured result. | timing.c and the timing command provide jitter, execution-time and CPU-load instrumentation. INT-03/INT-06 define tests; no board timing results are attached. | Measure motion, vehicle, barcode, hump and 1 kHz IR ISR execution maxima plus total load during telemetry, queue load and broker/AP loss. Jitter ≥2 ms fails NFR1. Re-test any priority change. | Buddy 2 with Buddy 1 |
| Task interaction; MQTT 6.1; FR11/NFR8 | Only cyw43 calls lwIP; other tasks use non-blocking rings. This avoids direct producer blocking, but scheduling interference still needs A2 validation. | At d5f5199, re-join code and host tests exist. Re-join starts after 3 s of link loss, then every 5 s. wifi_test.py is available; board result tables remain blank. | Measure hook cost/jitter. Justify flat retries, 15 s no-IP recovery and blocking radio restart gated to IDLE/STOPPED. FR10 still lacks reliable owner-confirmed applied replies. Full topic/schema documentation remains open. | Buddy 1 |
| Barcode 6.4; NFR2; A6/A11 | 1 kHz capture and decoding from the 29th element. At ≤300 mm/s at the sensor, a 3 mm bar lasts ≥10 samples. | Week 6 reports 1000/1000 synthetic frames and the sample *A* decoding. Code caps speed= at 300 mm/s. This supports decoder logic, not a board pass. | No attached contrast/track results. The turn/sensor-offset speed bound is missing. Measure placement and loaded latency; test 100, 180, 300 mm/s, both directions and bends. Resolve sensor-side coverage after U-turns. | Buddy 3 |
| IMU pins; hump 6.5; NFR6; A7/A12/A16 | Week 6 proposed LSM303DLHC, a front-contact estimator and a 10 mm trigger, targeting ±5 mm. Accuracy was not demonstrated on the car. | Commit 6f1bb86 ports the driver to LSM303D at 0x1D; TEST_IMU includes a scan. Settings: accel 100 Hz ±2 g, mag 50 Hz ±4 gauss. Physical identity/run evidence is still needed. | terrain.c still uses ds*sin(pitch) and a 6-degree trigger. Freshness flags/three-failure stop are absent. Measure L, noise, leading contact and slip. Test real humps; define reporting if NFR6 fails. Revalidate A16 rather than automatically discard its ±25% criterion. | Buddy 4 |
| Obstacle 6.6; FR7/8; NFR5; A9/A10 | OUT_OF_RANGE is clear only if A9 validates the installed no-target pulse. Two FAULTs or front data older than 200 ms should stop forward travel. | 2026-10-16 screenshot: 4 VALID (293-301 mm), 6 OUT_OF_RANGE, 0 FAULT; timeout-path widths 44-46 ms. Scan reports 196 mm distance/137 mm width, without confirmed servo motion or ground truth. | A9 remains open. Repeat controlled no-target/range tests and verify voltage/servo orientation. Monitor loses result class/freshness; whole aim/settle/ping locking and bypass leg checks are missing. Budget up to 12 s scans, 50 mm PASS steps and two re-plans; test corners and watchdog progress. | Buddy 5 |
| System statechart 5; FR12-14; NFR3/9 | Proposed queue-independent STOP latch, braking within 100 ms, CANCELLED completion, 100 ms velocity expiry and a 250 ms progress-gated watchdog. Reset starts braked. | These are Week 6 design contracts. Code has command IDs and stationary-only tuning, but not the complete fault/STOP design. Lecturer feedback explicitly distinguishes design quality from implementation. | STOP is still polled and can select calibration while idle. No latch/CANCELLED/expiry/watchdog. Indefinite waits and a 600 s move cap conflict with NFR9. Consolidate fault detection, latency, response, recovery and reason strings into the A14 checklist; inject every path before claiming readiness. | Buddy 5 with Buddies 1/2 |

## Shared Resource Allocation Matrix

Enter at least 3 shared resources, or more if the project has additional shared buses, timers, interrupts, actuators, sensors, queues, event groups, or shared data. A resource is “shared” if more than one task, module, interrupt, or buddy can affect it.

| Shared resource | Used by task(s) | Owner task / module | Access type | Contention risk | Mitigation | Evidence needed |
| --- | --- | --- | --- | --- | --- | --- |
| Sonar + servo GP6/7/12 | obstacle monitor, vehicle scans, standalone test | Buddy 5; sonar | Aim, settle, ping | Servo moves between front check and ping; monitor may already be mid-ping. | Current lock serialises pings and ≥60 ms spacing only; TMO_FEVR is unbounded. Extend ownership over aim+settle+ping with bounded, STOP-aware acquisition; avoid recursive locking. | INT-04: log angle, lock and trigger times; no off-centre front samples, ≥60 ms trigger spacing; test contention/STOP. |
| Motor PWM4/5 GP8-11 | vehicle requests; motion; standalone tests | Buddy 2; motion | Motion requests; PWM writes | Competing requests or stale velocity can keep the car moving. | One mission motor owner; motor.c performs PWM writes. Separate standalone tests. Newest request replaces old; 100 ms velocity expiry still needs implementation. | TEST_MOTION; suspend vehicle during velocity mode and verify expiry/braking; check reversal/coasting and signs. |
| CPU / scheduling | usb 3, cyw43 4, motion 5; remaining tasks/ISRs | Kernel scheduling; Buddy 1 poll work | Pre-emption | Network/re-join work or floating-point processing delays control/STOP. | Keep work bounded; measure before changing priorities. Jitter must be <2 ms. cyw43 below motion is a proposed fallback requiring regression tests. | A2: task/IR ISR maxima, total load and jitter under 5 Hz telemetry, queue load and broker/AP loss; measure gated restart separately. |
| IO_BANK0 vector | Encoder GP2/3; echo GP7; STOP GP21 planned | Shared HAL IRQ | Edge callbacks | Long callbacks or filtering reject/delay valid encoder/echo edges. | One short dispatcher timestamps once. Encoder speed uses same-edge periods over four slots. Validate the 300 us filter against actual shortest half-period, including unequal slot duty. | Add GPIO ISR duration measurement; compare echo distance with wheels stopped/running (±10 mm); log encoder half-periods at maximum speed. |
| I2C0 GP4/5 | imu in mission; scan in TEST_IMU | Buddy 4; imu | Read and write | Another user/long transfer delays samples; old data may appear fresh. | Keep one application owner and 10 ms transfer bounds. Freshness/data-ready and three-consecutive-failure reporting remain design work. | Capture identity/scan/settings, then 5-minute drive and stale/failed-read injection; verify proposed fault stop. |
| Shared status data | Provider tasks/ISRs; vehicle, imu, telemetry | Each provider | Publish, copy, read | Torn reads on SMP; long critical sections delay ISRs; missing timestamps hide stale data. | Short IRQ-protected snapshots on one core (A13). SMP guard is missing. Obstacle exposes a scalar; add class, time and freshness to obstacle and IMU contracts. | Check single-core build and critical-section cost; inject stale data and verify consumer handling. |
| STOP latch (planned) | GP21, USB/UART/MQTT; motion and vehicle | Vehicle clears; all sources set | Set, read, clear | STOP lost behind queues or racing START; design not yet implemented. | Queue-independent set paths; only accepted START clears after health checks. STOP must cancel work and never start calibration. | A14: 20 injections/source in each state with full queues; measure ≤100 ms braking, cancellation and recovery. |

## Subsystem Interface and Code Structure Map

Enter at least one interface or module per buddy. Each row should identify who provides the module, who consumes it, what inputs and outputs it exposes, and how it will be validated.

| Module / file / interface | Provider | Consumer(s) | Purpose | Inputs | Outputs | Interface rule | Validation rule |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mqtt_bridge.c / mqtt_lwip.c (IF-05) | Buddy 1 | cyw43, telemetry, dashboard | Network exchange | JSON ≤512 B; broker messages | Root: picocar/<i d>. Out: telemetry, event, heartbeat, status. In: cmd; link counters. | Only cyw43 calls lwIP. Rings do not block producers; full rings drop/count. | S1-U01; S1-U03: 10 min, ≥4.5 Hz, drop=0, RTT <100 ms. Record all topic schemas. |
| command.c / telemetry.c -> vehicle.h (IF-02) | Buddy 1 | vehicle; motion/line gains | Parse commands | Text command, optional #id | Vehicle request; reply ID, ok and error; gains report | Tune only in IDLE or STOPPED; speed 60-300. STOP can wait behind rings. Reliable retries and owner-confirmed applied replies are missing. | S1-U07/U08; full queues, lost replies, duplicate IDs and real application checks. |
| motion.h (IF-03) | Buddy 2 | vehicle; TEST_MOTION | Move wheels | Motion requests; encoder counts | PWM; distance, heading, speed; move result | One motor owner. Waits may be indefinite; move cap is 600 s. NFR9 requires ≤10 s and STOP cancellation; not implemented. | Five 500 mm runs: ±10 mm. Five 90-degree turns each way: ±5 degrees. A3: motion logs. |
| ir_sensor.c / line_follow.c | Buddy 3 | vehicle | Read line | GP26-28 ADC at 1 kHz; calibration | IR snapshot; ON, LOST, JUNCTION; steering | No direct motor calls. LOST after 150 ms; junction after 10 mm. Calibrated contrast must meet A5. | S3-U02: ≥50% ADC contrast per sensor; course and window lighting and motor-noise logs. |
| barcode.h (IF-01) | Buddy 3 | vehicle | Read code | 64-entry width ring from ISR | LEFT, RIGHT, STRAIGHT, U-TURN; other symbols report only | Four-message buffer, non-blocking poll; unused command discarded after 400 mm. Sensor-speed bound in turns still missing. | A6: ≥90% correct, 0 wrong at 100, 180, 300 mm/s. A11: post-U-turn placement and latency. |
| imu.h / terrain.h (IF-07) | Buddy 4 | vehicle, telemetry; motion supplies distance | Read IMU | LSM303D I2C0 0x1D; motion distance | Tilt, rate, hump and impact counts; I2C errors; no freshness flags | imu owns I2C0. Current pitch integral differs from the front-contact design. Freshness and three-failure stop remain missing. | NFR6: every error ≤5 mm, five passes per hump per speed. A7/A12 require real tests. |
| ultrasonic.h / obstacle.c / avoidance.c (IF-04) | Buddy 5 | vehicle; monitor | Range and plan | TRIG/ECHO, servo angle | Ping class, width, range; front scalar; profile and plan | Current lock covers ping only. Whole transaction, bounded wait and fresh fault status remain gaps. Clear space requires A9 validation. | S5-U02/04: box, servo and no-target logs. A10/INT-04: timing and shared use. |

## Week 6 Assumption Closure

Enter at least one assumption per buddy, preferably copied from Week 6. Each row should show whether the assumption is still open, validated, deferred, or converted into a risk.

| Week 6 assumption ID | Original assumption | Current status | Evidence collected | Remaining risk | Next action |
| --- | --- | --- | --- | --- | --- |
| A1 (Buddy 1) | Lab AP and laptop broker reachable without isolation; RTT <100 ms. | Needs validation | Week 6 host/sim_car evidence. Re-join and diagnostic code now exist; no attached 10-minute board result. | AP isolation, loss/recovery and reply delivery remain unverified. | Run S1-U03-U06 on the car; record link, drop and recovery results and commit. |
| A2 (Buddies 1/2) | usb/cyw43 interference keeps 20 ms control-loop jitter <2 ms. | Open | Timing instrumentation exists; no board workload result attached. | Control/STOP delay; M0+ soft-float CPU cost unmeasured. | INT-03/06: task and IR ISR maxima, jitter and total CPU load under telemetry, queue load and broker/AP loss. Investigate ≥2 ms; re-test any change. |
| A3 (Buddy 2) | 40 counts/rev; clean edges ≥300 us apart up to 300 mm/s; small slip; braking ≤40 mm from 300 mm/s. | Open | Code uses rising-to-rising speed periods averaged over four slots. M2 wiring correction is documented; no attached A3 board results. | Unequal slot duty, coast/reversal sign and slip corrupt odometry and stopping budget. | Count 10 hand turns; measure both half-periods at maximum speed/filter margin. Run distance/turn tests; measure coast, reversal and braking at 180 and 300 mm/s. |
| A6 (Buddy 3) | Barcode-sensor speed ≤300 mm/s, variation within ±20% over a 141 mm barcode. | Needs validation | Week 6 reports synthetic decoding. Code caps cruise commands, not offset-dependent sensor speed in turns. | Missed/wrong codes in turns or under noise/load. | Measure x_bc/y_bc; implement NFR2 bound; 20 passes at 100, 180, 300 mm/s, both directions and bends, ≥90% correct and 0 wrong. |
| A7 (Buddy 4) | Pitch noise ≤1 degree; L known ±1 mm; two-contact model fits chassis. | Needs validation | Week 6 simulation only; feedback cites 1.6-4.6 mm error. LSM303D port exists; intended estimator still absent. | Unknown noise, contact direction and slip; real height may fail ±5 mm. | Measure L/noise and leading contact both directions. Reconcile estimator, test flat floor and five passes per hump per speed. Report NFR6 unmet if accuracy fails. |
| A9 (Buddy 5) | 3.3 V module ranges ≥50 mm box over 20-400 mm; recognisable no-target long echo; servo 0 degrees right. | Needs validation | Mixed screenshot: 4 VALID (293-301 mm), 6 OUT_OF_RANGE, 0 FAULT. Timeout widths 44-46 ms; target conditions/servo motion unrecorded. | Range, echo-voltage and no-target discrimination unverified. A pulse that rises/stays high can appear clear. | Identify module. 20 pings/range at 20, 150, 300, 360, 400 mm: ≥95% valid, ±10 mm. Separate no-target/edge test. Decide 5 V + divider fallback; retest. If faults/no-target remain indistinguishable, revise clearance method. |
| A10 (Buddy 5) | ≥220 mm beside obstacle; line continues; retreat path, turns and calibration spin clear. | Open | Week 6 proposed geometry/checks only; no timed course bypass result attached. | Corners outside cone, collisions, missed rejoin, course-time overrun or watchdog timeout. | Five runs/placement: clearance, turn sweep, retreat/rejoin and total time. Budget ≤12 s scans, 50 mm PASS steps and two re-plans; verify progress/watchdog behaviour. |
| A11 (Buddy 3) | Barcode stays on sensor side in each reading direction; junction follows by 30-400 mm. | Open | Host decoder evidence does not verify physical course placement. | Sensor side flips after U-turn; code may be unreadable/late/expired. | Inspect every travel direction now; measure loaded decode-to-junction latency at 180 and 300 mm/s. If another sensor is needed, validate DOUT GPIO/ISR allocation and timing. |
| A12 (Buddies 4/5) | Humps ≥15 mm (10 mm trigger plus simulation margin); sonar does not treat hump as obstacle. | Open | Week 6 simulation only; no smallest-hump/sonar trace. Current estimator differs from design. | Low humps missed or false obstacle avoidance. | Measure smallest hump; after estimator reconciliation, log pitch, height and sonar at each speed. Lower trigger only with A7 flat-floor evidence. |
| A14 (Buddies 1/2/5) | STOP brakes within 100 ms under load in every state; stalled task stops car within 500 ms. | Open / blocked | No A14 trace. Latch, CANCELLED, velocity expiry and watchdog are missing. | Missed STOP, continued motion during scans/stalls or unsafe restart. | Consolidate one fault checklist: detection, latency, response, recovery and reason for STOP, stale velocity, watchdog, sonar/IMU failure, wait timeout and path/line failure. Implement gaps; 20 STOP injections/source in each state with full queues; test reset braked and START health checks. |
| A16 (Buddy 4) | Motor-running magnetic magnitude stays within ±25% of calibrated course value. | Needs validation | LSM303D configured 50 Hz ±4 gauss; no motor-running field results attached. | Magnetic disturbance corrupts heading/turn classification. New range alone does not invalidate the relative criterion. | Calibrate axes/offsets; log ¦B¦, heading and both turn rates across laps/turns. Check freshness and original ±25%; justify any revised limit against required accuracy. |

# Version B: Live Zoom-Updated Version

Instructions • During the recorded Zoom session, update this section live or immediately from discussion notes. • Record what changed from Version A and why. A change without justification is weak evidence. • Every buddy must contribute at least one challenge, defence, trade-off, or correction. • Any use of AI during the Zoom session must be screen-shared and visible to all participants at the point it is being discussed. • If AI-generated content is introduced during the live discussion, the team must explain the prompt used, the output received, and why the output is being considered. • AI suggestions must be challenged, validated, refined, or rejected through human engineering discussion; “AI said so” is not sufficient justification.

Acceptable evidence examples: GitHub commit link, short test log, video timestamp, oscilloscope or serial monitor screenshot, pin test result, task timing measurement, AI prompt and critique, meeting decision note, or photo of hardware wiring.

AI-use declaration: Do not ask AI to complete this template on behalf of the team. Use AI as a reviewer, challenger, or brainstorming partner. The team confirms that all final entries were reviewed, corrected, and accepted by the team, and that every member can explain the decisions related to their subsystem.

## Buddy Contributions During Zoom

Each buddy must complete one row. The contribution must be a real technical challenge, defence, trade-off, or correction raised during Zoom, not only an attendance statement.

| Buddy | Key challenge, concern, or argument raised | Challenge received | Decision or compromise reached |
| --- | --- | --- | --- |
| Buddy 1 | Raised that JSON telemetry and heartbeat must become binary so | Raees (Buddy 5) challenged that all the faults in one read and who clears the bits when the fault is gone. | Fault bitmask held in a uT-Kernel event flag (atomic set / clear, safe from ISRs), one writer per bit, vehicle reads every bit with one tk_ref_flag call. |
| Buddy 2 | Raised that packing encoder data shared with the encoder ISR would cause a read-modify-write race, because the M0+ has no atomic RMW (LDREX/STREX). Proposed keeping ISR-written counters as single-writer 32-bit words and packing only the motion status (~56 B → ~20 B) | Max (Buddy 3) challenged how would reducing the bits used for encoder counts and motor speed without causing overflow or losing the precision PID control needs | Each field’s range, units and precision are defined first, and its bit width is chosen from those. The raw ISR counter stays a 32-bit unsigned word, so wraparound is handled by computing deltas with unsigned subtraction. Only the per-period tick delta and speed (mm/s) are packed, and distance saturates instead of wrapping. PID calculations stay full width; only the setpoint and output are packed at interfaces. To be verified by host pack/unpack tests at min, max and out-of-range values, plus speed step response and timing.c max_us before and after. |
| Buddy 3 | Proposed storing barcode widths in 1 ms sampler ticks to reduce memory use. Defended rejecting over-range elements so a stopped or slow-moving car cannot cause a long bar to wrap into a false short bar and produce a wrong navigation command. | Lennon(Buddy 4) Asked what happens if reduced-width barcode timing overflows while climbing a hump or stopping over a bar. Challenged whether rejecting slow readings is acceptable and whether encoder-distance measurements are needed. | Proposed an 8-bit width field with 255 reserved as an over-range marker, plus a 1-bit bar/space flag in a 16-bit ring entry. Preserve the existing ≥250 ms white-gap framing rule; reject an over-range black element and reset the frame safely. Keep payload-before-head publication and synchronisation. Expected ring storage falls from approximately 512 to 128 bytes, subject to implementation measurements. Test each barcode 10 times at cruise and slowest hump speed before and after packing, including stopping over a bar. If slow-speed rejection is excessive, evaluate distance-based widths with Buddy 2, accounting for encoder resolution, wraparound and wheel slip. |
| Buddy 4 | The hump height is integrated at full width inside my task, because rounding 0.06 mm per step would lose it. My impact signal is set by me and cleared only by the vehicle controller, so it can't vanish before the stop has been commanded. | Levia (Buddy 2) asked does rounding also hit the turn rate used for straight-line correction? At 1 °/s resolution, a 0.5 °/s drift reads as zero and the car curves off the line unseen. And who clears the impact bit? If it clears on a timeout before the motion task reads it, a collision could be missed. | Turn rate is packed to ±250 °/s at 0.1 °/s (13 bits signed) so small drift stays visible, and heading is published separately at 0.1° (12 bits). Buddy 2 uses that heading for straight-line correction instead of integrating the packed rate. The IMU only sets the impact bit, and the vehicle controller clears it after commanding the stop, so no collision can be missed. We'll check both with a 2 m straight-drive drift test (within [X] cm) and a collision-stop test (within [Y] ms), before and after packing. |
| Buddy 5 | With correction to the fault-flag design, the vehicle reads the flag every 20ms, so it only sees faults present at that moment. A fault that is set and cleared between 2 reads is never seen, and a hug owner can’t set its own bit. This then already happens in my code: STOP is now latched by an interrupt because a short tap could be missed between polls, and the vehicle judges sonar “Stale” the 300 ms, from the age of the last reading. | Kent (Buddy 1): reading the flag and then five counters break the single snapshot. However the counts only ogo up, so a change after the read is caught in one loop which is 20ms later. Each count is one 32-bit word with one writer, on a single core, so it can't be half-read. A fixed hold time would have to cover the vehicles longest gap between reads which is about 100ms during a scan | Keep the flag for current faults, with one writer per bit. Each owner adds a fault count and a last update time. The vehicle catches short faults from the counts and dead owners of the age, and it decides the action for each fault. Measure the cost in timing.c and add it to the A14 fault checklist. |

## Live Decision Change Log

Enter at least 3 changes AND 3 confirmed no-change decisions from the Zoom discussion. Include the change type and evidence link or timestamp whenever possible. One row may state “No change” if the team defended the Week 6 design successfully.

| Original idea / Week 6 design | Updated decision | Change type | Reason for change | Evidence link / timestamp | Owner / follow-up |
| --- | --- | --- | --- | --- | --- |
| MQTT telemetry, heartbeat and commands as JSON text | Binary payloads with explict masks and shifts, fixed byte order and a version byte | Major redesign | Requirement: 268 > 19 B (AI estimate); bit-field layout is compiler-defined, so not used on the wire | 12:05pm in the recorded meeting. | Buddy 1: dashboard decoder |
| Fault flags spread across struct. | One fault bitmask (stall, encoder, IMU, Wi-Fi loss) as uT-Kernel event flag. | Simplified | Several writers including ISRs, event flag set / clear is atomic so vehicle can wait on it without being affected externally. | 12:05pm | Buddy 5 defines the bits, 2,3,4,5 will set their own. |
| Hardware and pins | No change | No change | The change is software only | 12:13 pm | All |
| No gyro, so no double-integrating acceleration. Tilt from the accelerometr, turn rate from the encoders and magnetometer | Kept | No change | The LSM303D has no gyro either, so the chip swap does not change this. Double integration drifts with the square of time | 12:20pm | Buddy 4 |
| MQTT TX ring 6x512 B | 6 x 32 B slots | Simplified | Binary messages are 19B; largest save in firmware. | 12:23pm | Buddy 1 |
| Obstacle monitor runs every 70ms at priority 9, below motion and vehicle | Keep priority 9 | No-change | The task that runs more often gets higher priority. The echo is then timed in the interrupt, so delay affects when the result arrives, not the distance. A stared monitor is caught by the 300ms stale stop. | 12:27PM | Buddy5 with Buddy2: Confirm in the A2 timing run |

## Resource and Assumption Review

| Item challenged | Evidence presented | Decision made | Impact on other buddies |
| --- | --- | --- | --- |
| MQTT TX ring sizing | AI Estimates: 3.1 KB static, binary messages are around 19 B only | Shrink to 32 B slots | Every buddy’s fields travel within telemetry |
| The 32-bit encoder counter never needs overflow handling | At ~[N] ticks/s, 2³² ticks takes years; unsigned subtraction handles a wrap anyway | Keep 32-bit unsigned; deltas by unsigned subtraction; distance saturates | Only Buddy 2 |
| Kernel event flag: shared fault bitmask – set/clear is atomic and safe from ISRs; one writer per bit | UT-Kernel are atomic system calls; 16 event flags available in kernel config. | One event flag holds stall, encoder, IMU, and Wi-Fi-loss bits. Buddy 5 defines bit map; each owner sets and clears only their own bit. | Buddies 2, 3, 4 and 5 each change their fault bit. Buddy 5’s vehicle task reads all bits with one call. |
| Hardware: pins and wiring. Assumption: the change is software. | Requirement covers data representation only; pin table and wiring unchanged. (12:13pm in meeting note) | No Change. | None. |
| Sensor: LSM303D (No gyro) Assumption: tilt from the accelerometer and turn rate from encoders + magnetometer are enough. | The LSM303D hjas no gyro; double-integrating acceleration drifts with time squared. (12:20) | Kept. Only field widths for pitch, roll and hump height change. | Buddy 2: encoder distance still feeds the hump estimator, so the distance field width affects buddy 4. |

## Risk Assessment and Subsystem Test Plan

### Integration Risk Register

Enter at least 3 integration risks. Include at least one timing or scheduling risk, one hardware or wiring risk, and one interface or team-integration risk.

| Risk ID | Risk | Cause | Impact | Likelihood | Severity | Mitigation | Owner | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1 (Hardware / wiring) | IMU drops out mid-run (loose Grove cable, vibration) and the car keeps using old values | b_ok is only set once, at boot, and never updated. A failed read only increments i2c_errors; the snapshot keeps its last pitch and impact values, with no freshness flag. | Missed humps or impacts and wrong terrain events, with nothing to show the IMU has failed | Medium | High | Add a sequence/freshness flag; set FAULT_IMU after 3 consecutive failed reads and let the vehicle task decide whether to stop; test by unplugging the IMU mid-run | Buddy 4 (vehicle reaction: Buddy 5/integration) | Open |
| R2 (Timing / scheduling risk) | IR sampling and packed-data processing delay motor control. | Packing operations or long critical sections in the 1 kHz IR ISR increase execution time and delay the motion task. | Motor-loop jitter reaches or exceeds 2 ms, affecting steering and speed control. | Medium | High | Keep ISR work and critical sections short. Measure ISR execution time and motor-loop jitter before and after packing under combined IR, barcode and network load. | Buddy 3 and Buddy 2 | Open |
| R3 (Interface / team integration) | Packed barcode events are interpreted incorrectly by the vehicle controller or telemetry. | Producers and consumers disagree on bit positions, units, field sizes or invalid-value markers. | Wrong navigation command or misleading telemetry. | Medium | High | Agree on one documented format and shared encode/decode functions. Update consumers together and test every command plus invalid values | Buddy 3, Buddy 1 and vehicle-controller owner | Open |

### Subsystem Test Plan

Enter at least 4 cross-subsystem tests. Each test must involve the work of at least two buddies and must include a success criterion and evidence type, such as a video, serial log, screenshot, timing result, or commit link. At least TWO test should involve timing, safety, or shared-resource concern.

| Subsystem | Test case | Success criteria | Evidence required | Owner | Status |
| --- | --- | --- | --- | --- | --- |
| Shared resource: GPIO interrupt | Box at 300mm, 20 pings with the wheels stopped, then with them spinning | Within +- 10 mm of each other; no front pings while the servo is off-centre | Serial log like the angle , trigger time, mm measurements) | Buddy 5 and Buddy 2 | Open |
| Safety: Obstacle and Motion | Block the ultrasonic sensor while driving at max speed | Car stops within a set range | Video or Serial log | Buddy 5 and Buddy 2 | Open |
| Shared Resource: Motion and Encoder ISR | Encoders at maximum edge rate while the motion task and Buddy 4 read packed data for 5 min | No torn or lost counts vs a reference count | Serial log | Buddy 2, Buddy 4 | Open |
| Interface: Barcode and Telemetry | Drive over each test barcode; Buddy 3’s decoded character and navigation command are packed into the binary telemetry and decoded on the dashboard | Every barcode shows the correct character and command on the dashboard; payload smaller than the JSON version | mosquitto log + dashboard screenshot | Buddy 3, Buddy 1 | Open |

## Constraint Injection / Curveball Response

Instructions • Record the injected constraint exactly as given. • Explain how it affects all five buddies, even if the impact differs by subsystem. • A good response shows prioritisation, trade-off reasoning, fallback behaviour, and evidence-aware decision making.

| Injected constraint | Buddy affected | Immediate impact | Adaptation / compromise |
| --- | --- | --- | --- |
| Minimum-bit data representation: every application data field uses only the bits its valid range needs (bit-fields or masks and shifts) - Faults in one shared bitmask - MQTT telemetry and command in compact binary - Packed shared data accessed safely | Buddy 1 | High: JSON payloads will be converted into binary. Commands are also text so it also need to be binary format. The laptop dashboard parses JSON, rewritten to decode binary. | Bit-level binary frames build into one places by packing buffer with masks and shifts. Ensures only one task touches the buffer so packing it will not cause a race. This does not incur extra cost or risk as byte saving over simpler byte format. |
| Same constraints | Buddy 2 | Medium: motion status shrinks from ~56 B to ~20 B when packed into their valid ranges. The real risk being the encoder data that is shared witht he encoder ISR, packing it alongside other fields would make the ISR and the motion task read-modify-write the same word, and with no LDREX/STREX on the M0+, encoder counts could be lost or torn (R1) make the Then the encoder data is shared with an ISR. | Packing at the interfaces and keeping full precision inside the control loop. The motion status and motion commands are packed using masks and shifts: duty at 7 bits per wheel, 1 direction bit per wheel, motor state at 3 bits, and speed and distance quantised to their measured maximum ranges. The PID integrator and calculations stay full width in local variables, so packing adds no accumulated rounding . As a documented exception, the ISR-written encoder counters stay in their own aligned 32-bit words with a single writer, and the task takes a snapshot of them before packing. This trades a few bytes of RAM for lock-free atomicity, without adding interrupt-off sections that would delay encoder timestamps. |
| Change Requirement: Minimum-Bit Data Representation The robot car firmware must minimise the number of bits used to represent application data. Each data field representing application information must use only the number of bits required to represent its valid range, using C bit-fields or explicit masks and shifts where appropriate. | Buddy 3 | Proposed response: review IR readings, calibration/filter data, line states, barcode widths, counters and queue events. Current representations need range and precision checks. Packing risks overflow, reduced steering/decoding accuracy and races between the 1 kHz ISR and tasks. Shared interfaces and binary telemetry must change together. | Keep the sensors and 1 kHz capture provisionally. Prioritise raw ADC 0-4095 (12 bits), line state (2 bits), five navigation results (3 bits) and bar/space (1 bit). Use masks/shifts; bound remaining fields and preserve filter/PID precision. Keep ISR/task ownership explicit; protect shared read-modify-write operations. Agree event formats with the vehicle owner and binary telemetry with Buddy 1; verify timing with Buddy 2. Before Week 13, document ranges/bit widths, measure RAM and message sizes before/after, and repeat line/barcode tests plus loaded ISR/jitter measurements. Savings and correctness remain unverified. |
| Same constraints | Buddy 4 | Medium: All shared IMU fields must be repacked and two consumers (telemetry.c, vehicle.c) updated, but the change is confined to the IMU's output boundary. The sensor fusion and hump algorithms are unchanged, and the existing DI/EI snapshot already meets the atomic-access requirement. | Minimum-bit representation is applied at the IMU's output boundary: the shared status snapshot and MQTT fields are packed into bit-fields (about 16 bytes, from 84). Internal filter and fusion state stays float32, because quantising it would add noise to pitch and drift to the hump height, which would be a loss of required function. Accepted costs: 0.1° angle resolution, hump values clamped to a measured range, and a saturating 8-bit impact counter (a wrapping one would break reset detection in vehicle.c). Fallback: widen any field whose measured range exceeds its bits. Evidence: TEST_IMU logs before and after, sizeof of the snapshot, and TIMING_IMU before and after. |
| Same constraints | Buddy 5 | Medium: the sonar monitor sets and clears on bit. Vehicle.c reads the flag every 20 ms and decides what each fault does. Resource: one new kernel event flag shared by alll five buddies, a few µs per 20ms loop., and a few words of RAM. Static/dynamic: bit numbers and limits are fixed in code; bits are set and read while running. Room: yes, the loop is 20 ms, and he watchdog has 2.5x margin (101 ms bad gap vs 250 ms limit). New problems: a fault that clears between two reads is never seen. A hung task can not set its own bit. STOP has many sources, so it breaks the “one writer per bit” rile. Every buddy sets and clears their own bit. | Priority: stop the car on faults that leave it blind or out of control: sonar (2 FAULTs in a row), motors, and IMU (Week 6 rule: three failed reads). Wi-Fi loss is report-only, because the STOP button and watchdog still work. Agree this with Kent. Trade-off: keep the flag for current faults. Each task also keeps a fault count and a last-update time, so short faults and dead tasks are caught without a second writer. Fallback: if the flag read fails, or a task hasn’t updated for too long, go to STOPPED and give the reason. STOP stays in the vehicle’s own latch. Evidence: inject each fault while line following (unplug the sonar, block the IMU, drop Wi-Fi). Pass if the car goes to STOPPED with the right reason within 100 ms (the NFR3 STOP budget). Measure the flag-read cost in timing.c |

## Integration Readiness Self-Assessment

Complete all rows. Use Green only if there is evidence, Yellow if partly ready or untested, and Red if the area is unclear or blocking Week 10 integration.

| Readiness area | Status | Evidence | Concern |
| --- | --- | --- | --- |
| Hardware wiring known | Green | Pin table complete and unchanged by requirement. (decision log 12:13). Motor PWM working since d4c531c; right-motor direction fixed in the wiring (562105b). LSM303D confirmed at 0x1D by the TEST_IMU I2C scan (6f1bb86). | HC-SR04 at 3.3 V is not yet qualified (A9 open). Encoder counts per rev (40) unmeasured, and bit widths depend on it (R2). |
| RTOS task ownership clear | Green | Single owners in code at d5f5199: cyw43 is the only lwIP caller, motion the only PWM writer, imu the only I2C0 user. The call added one writer per fault bit, with Buddy 5 owning the bit map, and the telemetry task as the only builder of binary frames. | The new owners (fault bits, frame builder, packed snapshots) are agreed but not coded. |
| Shared resources identified | Green | Version A matrix (sonar + servo, PWM, CPU, IO_BANK0, I2C0, status snapshots, STOP latch). The call added the fault event flag and the TX ring (6 × 32 B), and kept the ISR-written encoder counters as separate 32-bit single-writer words. | The sonar lock still covers only the ping (TMO_FEVR). Safe access to packed shared data relies on single core (SMP=0); enabling the second core breaks it. |
| Critical statecharts implementable | Amber | Vehicle states exist in code (7 states, fit in 3 bits). Run_move polls STOP every 20 ms with a 10 s timeout. Statechart contracts set in Week 6. | S TOP latch, CANCELLED result, 100 ms velocity expiry and watchdog are not in the repo at d5f5199. STOP in IDLE still starts calibration. obstacle_scan() doesn’t check STOP (up to ~12 s). |
| Safety or fault handling defined | Amber | Defined in the call: fault event flag with one writer per bit, plus per-owner fault counts and last-update times; stop rules (sonar 2 FAULTs, IMU 3 failed reads, motors); Wi-Fi loss report-only; fault → STOPPED within 100 ms (NFR3). | Defined but not implemented or tested. No A14 trace; fault injection not yet run. IMU b_ok is set only once at boot (R1). |
| Week 10 partial integration feasible | Amber | Host tests 123/123 and all 9 firmware modes build with 0 warnings at d5f5199. Motors driving; bare-board Wi-Fi outage tests passed (not yet logged) | All TEST_LOG tables empty, and no board timing (A2 jitter < 2 ms) measured. The binary frames, dashboard decoder and per-buddy packing are new work that touches every subsystem, and field ranges must be frozen first. |

Updated readiness position before the Week 10 partial integration demo:

CSC05 is Amber overall for the Week 10 partial integration. Wiring, task ownership and shared resources are known and evidenced at d5f5199, and the minimum-bit change is software-only. It is applied at each subsystem’s boundaries (binary MQTT frames, packed snapshots and queue messages), while ISR counters and control maths stay full width, so the integration structure is unchanged. The blocking work is the safety layer: the fault event flag, STOP latch, CANCELLED result, velocity expiry and watchdog are designed but not coded, and no board timing or fault-injection evidence exists yet. Before Week 10 we will freeze each field’s range and bit width, implement the packing with host round-trip tests, run A2 timing and A14 fault injection before and after packing, and record RAM and message sizes for the Week 13 evidence.

# Version C: Final AI-Vetted Gap Check

Instructions • After the live discussion, give the updated Version B content to AI and ask AI to act as a strict embedded systems reviewer. • Ask AI to identify missing assumptions, weak evidence, unclear interfaces, untested failure modes, and resource conflicts. • For every AI-identified gap, decide whether to fix, reject, or defer it, and explain why.

| AI reviewer prompt used |  |
| --- | --- |

| AI-identified gap | Team response | Action taken | Evidence / reason |
| --- | --- | --- | --- |

# 5. Final Tribunal Summary

| Final question | Team answer |
| --- | --- |
| What is the strongest part of the current design? |  |
| What is most likely to fail? |  |
| What evidence supports readiness? |  |
| What did AI miss that human discussion revealed? |  |
| What did AI correctly identify after discussion? |  |